Homework 10

Study Questions

Commercial AVF 221

Name___________________________________  Date____________ Score_____

Kershner chapter 12

- What are 2 reasons manifold pressure for 65% power is less at higher altitude?

- To increase power on a constant speed prop equipped airplane the order of engine control is:

- What is the order to decrease power?

- Why is it important to remember to move the prop to high before landing?

- What is the job of the governor?

- Draw a governor showing an underspeed condition

Kershner chapter 13

- What is a wobble pump?

- What is the optimum ratio of fuel to air?

- What is the difference in leaning procedure for constant speed vs fixed pitch?

- List a couple of advantages injection systems have over carbureted systems

- List 3 disadvantages of a fuel injection system

Kershner chapter 14

- What is the normal psi of a hydraulic landing gear system?

- Why does slowing the plane make the emergency landing gear extension easier?

- Why is it bad practice to slow the aircraft to get the prop stopped when executing a gear up landing?

Kershner chapter 16

- Why should you always use full carburetor heat?

- Is carburetor ice more of a function of temperature or humidity?

- What are 2 ways aircraft designers use to circumvent impact icing?

- How does an unlocked primer cause a rough running engine?

- What is the time difference for engine restart for carbureted vs injected engines after switching from an empty fuel tank to a full one?

- What is the most common cause of spark plug fowling?

- If oil pressure drops to 0, what is your next step?

- What is the first requirement when an electrical fire occurs on climb out?

- Is it recommended to restart the engine after following the engine fire checklist?

- Why is it good practice to execute a traffic pattern to an emergency landing spot?

- How many Gs are possible if impact speed is 80kts?

For 20 feet? _____

For 40 feet? _____