Homework 13

Study Questions

Commercial AVF 221

Name___________________________________  Date____________ Score_____

TIME, SPEED AND DISTANCE

SPEED 	TIME 		DISTANCE

1. 125 KTS 	_____		524 NM

2. _____	2:30 		345 NM

3. 110 KTS 	1:40		_____

4. _____	0:24 		44 NM

5. 95 KTS	1:24		_____

6. 90 KTS	_____		1005 NM

FUEL CONSUMPTION

GPHOUR 	TIME 		TOTAL USED

1. 7.8 		3:20		_____

2. _____	4:50		62

3. 8.5 		_____		38

4. 10 		2:30		_____

5. 12 		_____		22

DISTANCE CONVERSION

NAUT 	STAT 		KM

1. 20		_____		_____

2. _____	48		_____

3. _____	_____		110

AIRSPEED CONVERSION

PRESSURE 							DENSITY

ALTITUDE 	TEMP 		CAS 		TAS 		ALTITUDE

1. 14,000 	5°C 		160		_____		_____

2. 20,000 	-20°C 		200		_____		_____

3. 8,000 	15°C 		150		_____		_____

ALTITUDE CORRECTION

INDICATED/

PRESSURE 	CALIBRATED 		STATION 	TRUE

ALTITUDE 	ALTITUDE 	TEMP 		ALTITUDE 	ALTITUDE

1. 10,500 	10,000		-20°C 		5,000		_____

2. 12,000 	11,000 	-30°C 		3,000		_____

3. 8,000 	7,600 		-15°C 		(unknown)	_____

FEET/MILE VS. FEET/MINUTE

GROUND 	FEET PER MILE 	FEET PER

SPEED 	REQUIRED 		MINUTE

1. 120 		350			_____

2. 100 		250			_____

3. 150 		300			_____

WIND

TRUE

WIND 	WIND 	TRUE 			TRUE 		GROUND

DIREC. 	VELOCITY 	COURSE 	TAS 	HDG 		SPEED

1. 240 		38 		300 		165	_____		_____

2. 040 	43 		150 		140	_____		_____

3. 330 	25		020		80	_____		_____

4. 110 		18 		260 		225	_____		_____

AIRBORNE WIND

TRUE 		TRUE 			GROUND 	WIND 	WIND

HDG 		COURSE 	TAS 	SPEED 	DIREC. 	VELOCITY

1. 320 		315 		140 	128		_____		_____

2. 175 		160 		150 	115		_____		_____

Kershner Chapter 17

Kershner has 20 problems listed, read the instructions and work through them.  Check your answers which are given on the last page of the chapter.

PHAK Chapter 16

- What is the wing tip bearing change time-distance check formula for time?

- What is the wing tip bearing change formula for distance?

- Draw a diagram of the wing tip bearing change method, representing a 10° bearing change

- Explain the isosceles triangle method for determining time/distance to the station

- Draw a diagram representing a 10° angle for the isosceles method.

- What is the rule of 60?

- Plan a cross country from MWH to Rosalia (72S) and back in Bonanza 82964. All appropriate boxes must be filled.

Wind @ 5,500 and 4,500 is 230 true @ 25kts

Temp @ 5,500 is -14°C, Altimeter setting 29.92

Magnetic deviation to 72S is +2, to MWH is -2

Bonanza takeoff weight 3,200lbs, full tanks

Use a power setting of 22” MAP and 2300 rpm (65% power)

Include time, fuel and distance to climb in the pilot notes section

Switch tanks at Rosalia

Time off MWH is 1800 zulu

10 minutes for touch and go at 72S