Homework 7

Study Questions

Commercial AVF 221

Name___________________________________  Date____________ Score_____

Weight and Balance Handbook

- List the 4 factors that are effected by improper loading in terms of efficiency

- What is the responsibility chain for proper weight and balance control (start with engineers)

- Who has the responsibility on every flight to know the max allowable weight and CG limits?

- What are they for the Bonanza?

- Can severe uncoordinated maneuvers cause structural failure?

- What must be done with weight in a high density altitude situation?

- Name some other factors that would require a reduction in weight to maintain safety

- For positive longitudinal stability to result where must the CG be located referenced to the center of lift?

- Name some problems with a forward CG

- Name some problems with an aft CG

- Which condition requires more tail down force?

- Name the 2 elements that are vital in the weight and balance considerations of an aircraft

Define:

- Arm

- Datum

- Moment

- Type Certificate Data Sheet

- MAC

- LEMAC

- TEMAC

- Zero fuel weight

PHAK (chapter 10)

- List the performance deficiencies of an over loaded aircraft

- When is a nose heavy loading condition most likely to cause problems?

- Who sets the limits for the CG location?

- Where do most manufactures locate the fuel tanks with respect to the CG?

- What FAR requires establishment of the ranges of weights and CGs?

- In what 3 documents may the manufacturer provide this information?

- Under part 91 are there specified requirements to conduct weight and balance calculations prior to each flight?

- Does the addition of nuts, bolts, washers and rivets require a weight and balance check?

- The Bonanza’s GW is 3,400lbs.  What maximum weight meets the criteria for negligible weight change?

- When are part 135 aircraft exempted from the 36 month weigh-in requirement?

- Work up a weight and balance for Bonanza 82964.

Given: Pilot 160lbs @ 89, Pass 150lbs @ 89, 1 Backseater (Mother-in-law) 300 @ 127, 93.5lbs in baggage compartment, T/O full fuel - 80 total 74 useable, fuel used 60 gallons

| Beech F-33A | Weight | Arm | Mom/100 |
| --- | --- | --- | --- |
| Basic Empty Wt | 2264.50 | 80.65 | 1826.42 |
| Front Seat Fwd Pos. Aft Pos. |  | 85 89 |  |
| Rear Seat Fwd Pos. Aft Pos. |  | 121 150 |  |
| Baggage |  | 150 |  |
| Fuel(_74___gal) |  | 75 |  |
| -Taxi T/O fuel | -12 | 75 |  |
| T/O Condition |  |  |  |
| Less fuel used |  | 75 |  |
| Landing Condition |  |  |  |

In on takeoff? ______

In on landing? ______

How far out on landing? ______

How much cargo must be shifted to the front seat passenger position (Arm 89) to bring the CG into limits for the landing configuration? _______ (show your W/S formula work below)

Are we still in limits for takeoff after the shift? _______

How much weight is left in the cargo area? ______