Homework 9

Study Questions

Commercial AVF 221

Name___________________________________  Date____________ Score_____

- What is the equation of state?

- Define:

Rho

P

TR

1716

- How does a change in temperature affect pressure and density?

- How does the airspeed indicator only measure dynamic pressure?

- What is position error?

- What is instrument error?

- Why are errors found in airspeed in high angle of attack and skids?

- Does the airspeed indicator account for changes in pressure?

- Does the airspeed indicator account for changes in density?

- Define calibrated airspeed

- Define equivalent airspeed

- Define true airspeed

- What happens to the difference between CAS and EAS as altitude and airspeed is increased?

- Does TAS equal ground speed in no wind conditions?

- Using your E6b find the TAS for:

IAS 130kts			IAS 130kts

Temp +15°C              		Temp +33°C

PA 10,000                  	 PA 1,000

- Why is it higher at 10,000?

- Under part 23 how close does TAS (standard day) have to be to CAS?

- On what pages in the Bonanza POH can we find airspeed and altimeter calibration/correction information for the Bonanza?

Define:

- True altitude

- Absolute altitude

- Pressure altitude

- Indicated altitude

- Density altitude

- When flying from high to low will the airplane be higher or lower than what is indicated on the altimeter?

- As pressure goes up what happens to indicated altitude?

- If you increase the numbers in the Kolesman window what happens to altitude?

- What is the difference between an encoding altimeter and a blind encoder?

- Why does the reading on the VSI lag?

- What is the average lifetime of a geomagnetic field polarity?

- When a turn is started from south on the magnetic compass, which way does the card turn?

- How many steps does Kershner have for swinging a compass?

- What is the principle of gyroscopic rigidity in space?

- Resistance to change is a function of what 3 items?

- Which of Newton’s laws back up the behavior of rigidity in space?

- What is a gimbal?

- How fast does the attitude indictor gyro spin?

- What are the pitch and bank limits on the older attitude indicators?

- What error is induced by aircraft acceleration on the attitude indicator?

- Which way is the axis of the gyro orientated in an attitude indicator?

- Which way is the axis of the gyro orientated in a heading indicator?

- How does the rotation of the earth cause errors in the heading indicator?

- What happens if a gyro is placed on a stand with the spin axis 90° to the pull of gravity?

- How does the turn coordinator and turn and slip indicator use precession?

- What is the angle of the spin axis of the turn coordinator?

- What’s the idea of tilting the axis in the turn coordinator?

- What internal workings drive the tachometer?

- What is inside the MAP that enables it to measure manifold pressure?

- What is a Bourdon tube?

- What is a thermocouple?

- What are the three components to an electrical fuel gauge?